# Adhesive bond failure investigation form

Field record for a bonded joint that failed in production, in a test or in service.

How to use this form If there is a safety signal (fall, leak, fire risk, exposure of people), follow the site emergency procedure first and record only the escalation here. Fill the sections in order: evidence on the failure surface and in process records disappears quickly. Fill every field or write unknown with a reason. Mark each process value as measured or specified. Photograph both failure surfaces with a scale and a label before parts are handled, cleaned or bagged. Name failure patterns from your copy of ISO 10365:2022 (or ASTM D5573 for fiber-reinforced plastic) and estimate percent area so the shares total 100. Handle cleaning agents, primers and adhesives on site only under the site rules and the product safety data sheet (SDS). This form records evidence. It does not decide the cause: list causes in section 7 as unconfirmed hypotheses.

# 1. Incident overview and affected scope

| Case or complaint ID |  |
| --- | --- |
| Date and time failure was found (YYYY-MM-DD hh:mm), found by |  |
| Date and time of this visit (YYYY-MM-DD hh:mm) |  |
| Customer, site, line or location |  |
| Part number, assembly and joint location |  |
| What happened (observed facts, one per line) |  |
| What was reported (with who reported it) |  |
| Quantity: failed / inspected / not yet inspected (count) |  |
| Safety signal (fall, leak, fire risk, exposure of people): present or absent |  |
| If present: escalated to whom, date and time |  |

Affected scope per group of parts (a group nobody has checked stays unconfirmed):

| Group of parts or lot | Quantity (count) | Period made or shipped | Verdict: affected, unaffected or unconfirmed |
| --- | --- | --- | --- |

# 2. Initial response and control measures

| Control measure (hold, quarantine, stop) | Scope | State: requested, approved or executed, with evidence | Release condition |
| --- | --- | --- | --- |

Evidence samples:

| Sample (failed part, unused part, retained adhesive) | Quantity | Held by | Date collected |
| --- | --- | --- | --- |

[ ] Failed parts bagged and labelled

[ ] Unused part from the same lot secured, or not available

[ ] Retained adhesive from the same lot secured, or not available

# 3. Failure surface

| Joint or part ID | Pattern designation (ISO 10365:2022 or ASTM D5573) | Share of bonded area (%, total 100) | Photo references (both surfaces) |
| --- | --- | --- | --- |

[ ] Both mating surfaces examined

[ ] Magnification used: no / yes (state type)

[ ] Photos taken before handling, with scale and label

Sketch of joint and failure areas:

# 4. Substrate and preparation

| Substrate 1: material, grade, coating or treatment |  |
| --- | --- |
| Substrate 2: material, grade, coating or treatment |  |
| Preparation procedure name and revision |  |
| Preparation done as written? (yes, no, unknown) and record |  |
| Time from preparation to bonding (h) |  |
| Contamination seen at the interface (oil, release agent, dust, moisture) |  |

# 5. Adhesive and cure

| Adhesive product type and name |  |
| --- | --- |
| Adhesive lot or batch (from label or record) |  |
| Expiry or shelf life status |  |
| Mix ratio specified and as dispensed (by weight or volume) |  |
| Mixing method (static mixer, hand, meter-mix) |  |
| Bond line thickness measured on failed part (mm, range) |  |
| Cure temperature: ambient or part (°C) |  |
| Cure time before handling and before load (h) |  |
| Each value marked measured or specified |  |

# 6. Service environment and load

| Service temperature range (°C) |  |
| --- | --- |
| Relative humidity range (%) |  |
| Load type (static, peel, vibration, impact) and estimated load |  |
| Fluids, chemicals or outdoor exposure |  |
| Age of joint at failure (days) |  |
| Load or event at the moment of failure, if known |  |

# 7. Open hypotheses

List every possible cause across substrate, preparation, adhesive, mixing, cure, design and service. Each stays unconfirmed until the check in the last column is done.

| Hypothesis (cause axis) | Record for | Record against | Check that settles it |
| --- | --- | --- | --- |

# Completed by

| Name and organization |  |
| --- | --- |
| Date (YYYY-MM-DD) |  |
| Report this form feeds (case or report ID) |  |

# Example (illustrative)

Illustrative example only. The part, quantities, readings and hypotheses are invented to show the format. They are not a real case or test data.

## 1. Incident overview and affected scope

| Case or complaint ID | BF-0001 |
| --- | --- |
| Date and time found | 2026-04-08 09:30, line inspector |
| Date and time of visit | 2026-04-09 14:00 |
| Part and joint | Bracket to coated steel panel, line 2 |
| Observed facts | 6 brackets off panels |
| Quantity | 6 failed / 240 inspected / 180 in stock not inspected |
| Safety signal | Absent |
| Affected scope | Stock lot on hold, 180, April week 14, unconfirmed |

## 2. Initial response and control measures

| Control measure (hold, quarantine, stop) | Scope | State: requested, approved or executed, with evidence | Release condition |
| --- | --- | --- | --- |
| Hold on stock | 180 assemblies | Executed, hold tag photo | Cause agreed in corrective action report |

| Sample (failed part, unused part, retained adhesive) | Quantity | Held by | Date collected |
| --- | --- | --- | --- |
| Failed parts | 6 | Quality lab | 2026-04-08 |
| Unused part, same lot | 1 | Quality lab | 2026-04-09 |
| Retained adhesive, same lot | Not available |  |  |

## 3. Failure surface

| Joint or part ID | Pattern designation (ISO 10365:2022 or ASTM D5573) | Share of bonded area (%, total 100) | Photo references (both surfaces) |
| --- | --- | --- | --- |
| B07 | Adhesive failure at panel coating side (write the designation as given in ISO 10365:2022) | 70 | L2-B07-panel.jpg, L2-B07-bracket.jpg |
| B07 | Cohesive failure (write the designation as given in ISO 10365:2022) | 30 | as above |

## 4 to 6. Process and service

| Substrate and preparation | Powder-coated steel panel; wipe and abrade, 30 h before bonding; abrade step not signed for 2 shifts (measured record) |
| --- | --- |
| Contamination at interface | None seen |
| Adhesive and mix | Two-part epoxy, lot on label, 2:1 as dispensed (dispenser log) |
| Expiry and mixing method | In date, static mixer |
| Bond line | 0.2 to 0.6 mm measured on two failed parts |
| Cure | About 12 °C, 4 h before load (line log, measured) |
| Service | 15 to 30 °C, 40 to 70 % RH, vibration from conveyor, indoor |
| Age of joint and load at failure | 21 days; load at failure unknown |

## 7. Open hypotheses

| Hypothesis (cause axis) | Record for | Record against | Check that settles it |
| --- | --- | --- | --- |
| Coating surface not prepared as written | Abrade step not signed for 2 shifts | Wipe step signed | Retest bonded coupons from held panels |
| Low cure temperature | 12 °C at cure, 4 h before load | Data sheet not yet compared | Compare with data sheet cure schedule |
| Mix ratio off | None yet | Dispenser log shows 2:1 | Check dispenser calibration record |
| Coating type changed | None yet | Same coating code on panels | Confirm coating supplier lot change |