# Certificate of analysis template for an industrial chemical

## Instructions

Use one certificate per lot. Every result on the certificate applies to the lot in section 2 only.

Fill in section 4 from the customer's specification first: copy each test, method number and edition, unit and limits from the specification revision the customer holds. Enter results afterwards.

Write each result as reported after the rounding stated for that test, in the same unit as the limit. Give the unit in the unit column, not inside the result cell.

State the comparison rule below the results table: rounding method or absolute method (ASTM E29), and any decision rule for measurement uncertainty agreed with the customer.

If any result is out of limits, the statement in section 5 names that test. Do not sign a general conformance statement over a failed result.

If you reissue another company's certificate, enter the original certificate number and laboratory in section 1 and attach a copy of the original.

Mark checkbox items by writing X inside the brackets: [X].

The example on the last page is illustrative. The product, specification and values are invented and are not limits for any product.

Delete these instructions before issuing the certificate.

# Certificate of analysis

## 1. Certificate identity

| Field | Unit or format | Entry |
| --- | --- | --- |
| 1.1 Certificate number | Text |  |
| 1.2 Issuing company and laboratory, address | Text |  |
| 1.3 Issue date | YYYY-MM-DD |  |
| 1.4 Original certificate number and laboratory (reissued certificates only) | Text |  |

## 2. Product and lot

| Field | Unit or format | Entry |
| --- | --- | --- |
| 2.1 Product name and grade (as on the order) | Text |  |
| 2.2 Product code | Text |  |
| 2.3 Lot or batch number | Text |  |
| 2.4 Date of manufacture | YYYY-MM-DD |  |
| 2.5 Retest or expiry date (if the product has one) | YYYY-MM-DD |  |
| 2.6 Quantity and packaging | kg, L or units; pack type |  |

## 3. Recipient and order

| Field | Unit or format | Entry |
| --- | --- | --- |
| 3.1 Customer name and receiving site | Text |  |
| 3.2 Purchase order number and line | Text |  |
| 3.3 Delivery or packing list reference | Text |  |

## 4. Results against specification

| Field | Unit or format | Entry |
| --- | --- | --- |
| 4.1 Specification number and revision | Text |  |
| 4.2 Date of analysis | YYYY-MM-DD |  |

| Test | Method number and edition | Unit | Limits (min / max) | Result as reported | Conforms (Y/N) |
| --- | --- | --- | --- | --- | --- |

| Field | Unit or format | Entry |
| --- | --- | --- |
| 4.3 Comparison rule | Rounding method / Absolute method |  |
| 4.4 Decision rule for measurement uncertainty (if agreed) | Text |  |

## 5. Statement and approval

5.1 Conformance statement (name the specification and revision; name any test out of limits):

__________________________________________________________________________________________

__________________________________________________________________________________________

__________________________________________________________________________________________

| Field | Unit or format | Entry |
| --- | --- | --- |
| 5.2 Approved by (name) | Text |  |
| 5.3 Role (outside production by default) | Text |  |
| 5.4 Date | YYYY-MM-DD |  |
| 5.5 Signature |  |  |

Checks before signing:

[ ] Lot number matches the batch record and the packing list

[ ] Product name and grade match the order and the TDS

[ ] Every test on the specification revision in 4.1 has a result

[ ] Every result has a method with edition and a unit matching its limit

[ ] Conformance column follows the rule in 4.3

[ ] Statement in 5.1 agrees with the conformance column

## Illustrative example (invented values, not a real case)

A hydraulic fluid lot. All names, numbers, dates and limits below are illustrative and are not limits for any product.

| Field | Illustrative entry |
| --- | --- |
| 1.1 Certificate number | CoA-2026-0412 |
| 1.3 Issue date | 2026-10-08 |
| 2.1 Product name and grade | Hydraulic fluid, ISO VG 46 |
| 2.2 Product code | HF46 |
| 2.3 Lot number | 26-1192 |
| 2.4 and 2.5 Dates | Manufactured 2026-09-30; retest 2028-09-30 |
| 2.6 Quantity and packaging | 16,000 kg in 80 drums |
| 3.2 Purchase order | PO 4500123 line 2 |
| 4.1 Specification | Customer specification QS-12 revision C |
| 4.3 Comparison rule | Rounding method |

| Test | Method | Unit | Limits | Result | Conforms |
| --- | --- | --- | --- | --- | --- |
| Kinematic viscosity at 40 C | ASTM D445-26 | mm2/s | 43.0 to 49.0 | 46.1 | Y |
| Water content | ASTM D6304-25 | % | 0.05 max | 0.03 | Y |
| Acid number | ASTM D664-24 | mg KOH/g | 0.10 max | 0.12 | N |
| Appearance | Visual |  | Clear and bright | Clear and bright | Y |

5.1 Statement (illustrative): Lot 26-1192 does not conform to customer specification QS-12 revision C: acid number 0.12 mg KOH/g against 0.10 mg KOH/g maximum. All other tests conform.