# Chemical compatibility worksheet

How to use this worksheet Use one worksheet per question: can this product be used with what it meets in this system? Fill in the sections in order. 1. Write every combination on its axis (coexisting chemical, wetted material or residue at changeover) with concentration (%), temperature range (°C, including start-up, cleaning and seasonal extremes) and contact time (h). Add neat contact in shared lines, bunds or spills as its own row. 2. Write the criterion, limit and source for each axis before you read any chart or test value. 3. Identify each product by component, CAS number and concentration band from section 3 of its safety data sheet, and each wetted material by grade. A trade name or a material family alone leaves the row unresolved. 4. Mark every value from a public chart, handbook or another site's experience as typical, with the conditions it was measured at. 5. Give each conditional verdict its limits, a sign to watch, an observation point and frequency, and a stop reading. 6. Mixing or bottle tests of chemicals that can react are hazardous. Before mixing, read sections 7 and 10 of both safety data sheets, start with small quantities and follow the site's written procedure (in the US, the employer's Chemical Hygiene Plan under OSHA 29 CFR 1910.1450 for laboratory use). Verdicts: C = compatible (evidence covers the conditions), CC = conditionally compatible (condition is a test or a limit), I = incompatible. Confidence: H, M or L. An illustrative example is on the last page, kept apart from the blank form.

Assessment ID: _________________________________________________________

Date (YYYY-MM-DD): _____________________________________________________

Prepared by: ___________________________________________________________

Reviewed by: ___________________________________________________________

Customer and site: _____________________________________________________

System: ________________________________________________________________

Product assessed (name, SDS revision date): ____________________________

Question (one sentence): _______________________________________________

# 1. Combinations and criteria

Combinations. One row per combination; neat contact gets its own row.

| ID | Axis (chemical, material, residue) | Other side | Contact point | Concentration (%) | Temperature range (°C) | Contact time (h) | Neat contact (Y/N) | Avoidable (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Criteria. Fixed before any value is read.

| Axis | Criterion (property) | Limit (value, unit, sign) | Source (customer spec, maker statement, or typical) |
| --- | --- | --- | --- |

[ ] Criteria and limits were written before any chart or test value was read.

[ ] Neat contact in shared lines, bunds or spills has its own combination row.

[ ] Every temperature range includes start-up, cleaning and seasonal extremes.

# 2. Compositions and grades

Products, coexisting chemicals and previous products (from SDS section 3 and section 10).

| Product | Component | CAS number | Concentration band (%) | Public or internal | SDS section 10 incompatibilities | SDS revision date |
| --- | --- | --- | --- | --- | --- | --- |

Wetted materials.

| Material and location | Grade (alloy, compound, polymer, coating system) | Record it comes from | Grade confirmed? (Y/N) |
| --- | --- | --- | --- |

[ ] Each product row uses the safety data sheet for the customer's jurisdiction.

[ ] Each material known only by its family is marked grade unconfirmed.

# 3. Verdict per combination

Combinations that can give off gas, heat or a hazardous product go first.

| ID | Deciding value (unit) | Limit | Test conditions vs customer conditions | Source kind and date | Typical? (Y/N) | Verdict (C, CC, I) | Confidence (H, M, L) |
| --- | --- | --- | --- | --- | --- | --- | --- |

Overall verdict (C, CC or I) and the combination that decides it: __________

[ ] No combination is called compatible beyond the concentration, temperature and contact time its evidence covers.

[ ] Every value from a public chart or handbook is marked typical.

# 4. Operating limits and signs to watch

Each conditionally compatible combination.

| ID | Limit (value, basis) | Sign to watch | Observation point | Frequency | Stop reading |
| --- | --- | --- | --- | --- | --- |

Each incompatible combination the customer can avoid.

| ID | Separation measure (dedicated line, tank, flush, segregated storage) |
| --- | --- |

[ ] If no combination is conditional or avoidably incompatible, this is stated here in one sentence.

# 5. Confirmation tests

| ID | Test and method | Conditions (°C, h, mixing ratio) | Acceptance criterion (from section 1) | Sample | Owner | Due date |
| --- | --- | --- | --- | --- | --- | --- |

Test that decides the overall verdict: _________________________________

[ ] Sections 7 and 10 of both safety data sheets were read before any mixing test.

[ ] Mixing tests follow the site's written procedure and start with small quantities.

[ ] Owners and due dates not yet agreed are marked to be agreed.

# 6. Limitations

| Open item (composition, grade, condition, typical value) | Verdict it holds up | How it will be settled | Who can settle it | By when |
| --- | --- | --- | --- | --- |

Changes that call for a new assessment.

[ ] Added coexisting chemical

[ ] Material change (gasket, seal, pipe, coating)

[ ] Higher temperature or new cleaning step

[ ] Changed dose, concentration or contact time

[ ] Revised safety data sheet for either side

[ ] New test result for a conditional row

Next review date (YYYY-MM-DD): _________________________________________

Issued by, date: _______________________________________________________

# Example (illustrative)

Every product, value, name and date on this page is invented to show how the worksheet is filled in. It is not data from a real system and not a verdict for any product.

Question: can Product A, an acidic cleaner, be used in a circuit that also receives Biocide B and has EPDM gaskets and 316L stainless steel pipe? (illustrative)

| ID | Axis | Other side | Contact point | Concentration (%) | Temperature range (°C) | Contact time (h) |
| --- | --- | --- | --- | --- | --- | --- |
| C-01 | Coexisting chemical | Biocide B, neat | Shared bund | 100 | 5 to 35 | Spill event |
| C-02 | Coexisting chemical | Biocide B, in circuit | Circuit water | 2 | 15 to 45 | Continuous |
| C-03 | Wetted material | EPDM gaskets | Dosing line | 2 | 15 to 45, cleaning 70 | Continuous |
| C-03N | Wetted material | EPDM gaskets, neat product | Dosing line | 100 | 15 to 45 | Continuous |
| C-04 | Wetted material | 316L pipe | Circuit | 2 | 15 to 45 | Continuous |

| Product | Component | CAS number | Concentration band (%) | SDS section 10 incompatibilities |
| --- | --- | --- | --- | --- |
| Product A | Phosphoric acid | 7664-38-2 | 10 to 20 | Strong bases, oxidizers (illustrative) |

| ID | Deciding value against limit | Verdict | Next step |
| --- | --- | --- | --- |
| C-01 | SDS section 10 of Biocide B lists acids as incompatible | I | Separate bunds and dosing lines |
| C-02 | No data at in-use dilution | CC | Bottle test at system ratio |
| C-03 | Supplier chart rates EPDM good at 23 °C, typical | CC | Immersion test at 70 °C, 168 h |
| C-03N | Supplier chart rates EPDM good at 23 °C, typical; neat product in dosing line | CC | Immersion test at 70 °C, 168 h |
| C-04 | Corrosion rate below limit at 50 °C, supplier test | C | Hold below 50 °C |

Overall verdict (illustrative): incompatible until C-01 is separated; then conditionally compatible pending the C-02 bottle test. Open item: gasket compound unconfirmed, settled by the maintenance planner.