# Corrosion test panel rating record sheet

## Instructions

Use one sheet per exposure run and inspection. Rate the control panel first.

Rate rust (ASTM D610), blistering (ASTM D714) and scribe creep (ASTM D1654 or ISO 4628-8) against the standards' own pictorial references, kept beside the panel. This sheet does not reproduce them. Record the scribe procedure and the method used to remove loose coating before measuring creep.

Write each rating exactly as the standard designates it. Keep ASTM and ISO 4628 ratings apart.

Fill section 2 before exposure starts. Do not change a criterion after panels are rated.

Leave a rating blank and give the reason in Notes rather than guess. Transfer entries to the Excel log the same day.

The example page at the end is illustrative only and is kept apart from the blank form.

## 1. Run and inspection

Exposure test and edition: __________________________________________________

Cabinet or site ID: __________________________________________________

Exposure start date (YYYY-MM-DD): __________________________________________________

Exposure hours at this inspection (h): __________________________________________________

Inspection date (YYYY-MM-DD): __________________________________________________

Rater (name): __________________________________________________

Second rater, if used (name): __________________________________________________

Lighting and viewing conditions: __________________________________________________

Substrate and pretreatment: __________________________________________________

Scribe type: __________________________________________________

Scribe method, procedure and removal: __________________________________________________

## 2. Acceptance criteria (as frozen before exposure)

Rust criterion (ASTM D610 grade or ISO 4628-3): __________________________________________________

Blister criterion (ASTM D714 size and density or ISO 4628-2): __________________________________________________

Scribe creep criterion (mm, mean or maximum): __________________________________________________

Date criteria were frozen (YYYY-MM-DD): __________________________________________________

Agreed by (both parties): __________________________________________________

## 3. Panel ratings

Rust distribution: S spot, G general, P pinpoint, H hybrid (ASTM D610). Judgment: met, borderline, not met, unassessed.

| Panel ID | Replicate group | Control? [ ] | System | DFT (µm) | Rust grade D610 (0-10) | Rust distr. [ ]S [ ]G [ ]P [ ]H | Blister size D714 | Blister density D714 | Creep mean (mm) | Creep max (mm) | Scribe rating D1654 (0-10) | Photo ref. | Judgment [ ]met [ ]bord. [ ]not met [ ]unass. | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 4. Run check and sign-off

[ ] Control panel rated first and within its known range

[ ] Cabinet records checked for deviations during this interval

[ ] Every panel photographed under the same light

[ ] Edges and hanger holes noted, not rated

[ ] Entries transferred to the Excel log

Signed (name, date): __________________________________________________

# Example (illustrative only)

Every value below is invented to show how the sheet reads. None comes from a real test.

| Panel ID | Replicate group | Control | System | DFT (µm) | Rust grade | Rust distr. | Blister size | Blister density | Creep mean (mm) | Creep max (mm) | Scribe rating | Photo ref. | Judgment | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P-001 | Control | yes | Reference R | 175 | 9 | S | 10 |  | 1.0 | 1.6 | 8 | P-001_1000h.jpg | met | Rated first |
| P-014 | SYS-B rep 2 of 3 | no | SYS-B | 180 | 9 | S | 8 | F | 1.2 | 2.0 | 7 | P-014_1000h.jpg | met | Hanger hole rust, not rated |
| P-015 | SYS-B rep 3 of 3 | no | SYS-B | 165 | 8 | P | 8 | M | 2.0 | 3.4 | 7 | P-015_1000h.jpg | borderline | At the limit; second rating under the same light asked |

Illustrative run: ASTM B117-26a, cabinet 2, 1000 h, criterion creep mean at most 2 mm. Substrate cold-rolled steel, zinc phosphate; single diagonal scribe to metal; scribe method ASTM D1654-24e1, removal per lab procedure LP-07.