Customer site readings form: cooling water and metalworking coolant

How to use this form Use one reading block for each sample point at each check. Copy a block for more checks. Write the agreed control range from your treatment program in section 2 before the first check. This form does not set ranges. Write every reading with its unit, the time measured and the method. A value without them cannot be compared with earlier checks. Measure pH and temperature at the sample point as soon as the sample is taken. pH can change within hours of sampling. Report conductivity in µS/cm at 25 °C with temperature compensation on. For coolant, write the refractometer scale reading and the factor from the fluid maker's data sheet, then the concentration. Read the safety data sheet (SDS) of every buffer, reagent and test kit before use. If a reading is outside the agreed range, recheck it, keep the first value, and complete section 5. Leave rows that do not apply to your system blank.

1. Site and system

| Site name |  |
| --- | --- |
| System tag (for example CT-1, central coolant tank) |  |
| System type: [ ] Open recirculating cooling water [ ] Water-mix metalworking fluid [ ] Other |  |
| Product or treatment program reference |  |
| Supplier contact (name, email) |  |
| Form period (from YYYY-MM-DD to YYYY-MM-DD) |  |

2. Agreed control ranges (from the treatment program)

| Reading | Unit | Low limit | High limit |
| --- | --- | --- | --- |
| pH | pH units |  |  |
| Conductivity | µS/cm at 25 °C |  |  |
| Coolant concentration | % |  |  |
| Inhibitor or biocide residual |  |  |  |
| Other: |  |  |  |

| Ranges agreed with (name) and date (YYYY-MM-DD) |  |
| --- | --- |

3. Instruments

| Instrument | Meter ID | Last calibration or check (YYYY-MM-DD) | Notes |
| --- | --- | --- | --- |
| pH meter and electrode |  |  |  |
| Conductivity meter |  |  |  |
| Thermometer or thermistor |  |  |  |
| Refractometer (factor: ______ from data sheet dated ______) |  |  |  |
| Residual test kit (name) |  |  |  |

4. Readings: one block per sample point per check

| Check 1: sample point name |  |
| --- | --- |

| Field (unit) | Agreed range | Reading | Out of range |
| --- | --- | --- | --- |
| Date and time sampled (YYYY-MM-DD hh:mm) |  |  | [ ] |
| Time measured (hh:mm) |  |  | [ ] |
| Sample temperature (°C) |  |  | [ ] |
| pH (pH units, to 0.01) and method |  |  | [ ] |
| Conductivity (µS/cm at 25 °C) |  |  | [ ] |
| Refractometer scale reading and factor |  |  | [ ] |
| Coolant concentration (%) = reading x factor |  |  | [ ] |
| Inhibitor or biocide residual (unit the program sets) |  |  | [ ] |
| Make-up meter (m3) |  |  | [ ] |
| Blowdown meter (m3) |  |  | [ ] |
| Appearance and tramp oil |  |  | [ ] |
| Operator and meter ID |  |  | [ ] |
| Remarks and action taken |  |  | [ ] |

| Check 2: sample point name |  |
| --- | --- |

| Field (unit) | Agreed range | Reading | Out of range |
| --- | --- | --- | --- |
| Date and time sampled (YYYY-MM-DD hh:mm) |  |  | [ ] |
| Time measured (hh:mm) |  |  | [ ] |
| Sample temperature (°C) |  |  | [ ] |
| pH (pH units, to 0.01) and method |  |  | [ ] |
| Conductivity (µS/cm at 25 °C) |  |  | [ ] |
| Refractometer scale reading and factor |  |  | [ ] |
| Coolant concentration (%) = reading x factor |  |  | [ ] |
| Inhibitor or biocide residual (unit the program sets) |  |  | [ ] |
| Make-up meter (m3) |  |  | [ ] |
| Blowdown meter (m3) |  |  | [ ] |
| Appearance and tramp oil |  |  | [ ] |
| Operator and meter ID |  |  | [ ] |
| Remarks and action taken |  |  | [ ] |

5. Out-of-range follow-up

[ ] Reading rechecked with a checked instrument (time of recheck: ______ )

[ ] Sample point, method and instrument are the same as the last check

[ ] Temperature recorded with the pH and conductivity readings

[ ] Refractometer factor applied and scale line was sharp, not diffuse

[ ] First value kept on the form; recheck written as a new line

[ ] Supplier told (date and time: ______ )

[ ] Action taken only as the treatment program says (write it in Remarks)

| What was seen and done |  |
| --- | --- |

6. Send back to the supplier

[ ] Completed reading blocks with every field filled or marked not applicable

[ ] Agreed control ranges (section 2)

[ ] Meter IDs and last calibration dates (section 3)

[ ] Photos of anything unusual (foam, deposits, diffuse refractometer line)

[ ] Laboratory sample label and time it left the site, if a sample was sent

[ ] Original sheet kept on site for the period your program sets

| Sent to (name) and date (YYYY-MM-DD) |  |
| --- | --- |
| Sent by |  |

Illustrative example (not part of the form)

All values below are illustrative. They are not targets or recommended ranges. Your control ranges come from your own treatment program.

| Check 1: sample point name | CT-1 basin return line (illustrative) |
| --- | --- |

| Field (unit) | Agreed range | Reading | Out of range |
| --- | --- | --- | --- |
| Date and time sampled (YYYY-MM-DD hh:mm) |  | 2026-10-20 09:15 | [ ] |
| Time measured (hh:mm) |  | 09:20 | [ ] |
| Sample temperature (°C) |  | 28.4 | [ ] |
| pH (pH units, to 0.01) and method | 7.8 to 8.6 | 8.42, glass electrode meter | [ ] |
| Conductivity (µS/cm at 25 °C) | 1500 to 2000 | 1850 | [ ] |
| Refractometer scale reading and factor |  | Not used (cooling water) | [ ] |
| Coolant concentration (%) = reading x factor |  | Not used | [ ] |
| Inhibitor or biocide residual (unit the program sets) | 3 to 5 mg/L as product | 4.1 mg/L as product | [ ] |
| Make-up meter (m3) |  | 15 230 | [ ] |
| Blowdown meter (m3) |  | 3 810 | [ ] |
| Appearance and tramp oil |  | Clear, slight foam at basin edge | [ ] |
| Operator and meter ID |  | AB, meters PH-02 and EC-01 | [ ] |
| Remarks and action taken |  | Make-up valve serviced 08:00. No action. | [ ] |