# Customer Trial Report

Report of an industrial product trial to the customer's approver: criteria fixed beforehand, conditions as run, results against each criterion, verdict and approved scope.

# Instructions

1. Write the report for the customer's approver. Keep the supplier's internal reasoning out of it. 2. Copy every criterion, threshold, method, comparison basis and freeze date from the trial plan unchanged. A criterion changed after a result was seen is a new criterion and needs a new run. 3. Fill in section 2 before section 3: a run that cannot be read is reported as a run to repeat, not judged. 4. Account for every planned sample: tested, excluded with a reason, or lost with a deviation number. 5. Margin (%) = (result - threshold) / threshold x 100 when a higher result passes, and (threshold - result) / threshold x 100 when a lower result passes. A positive margin reads as passing. This is a convention of this template; use the rule in your trial plan if it differs. When the threshold is zero or a range, leave the margin blank and judge the result against the threshold directly. 6. A criterion without a valid result keeps its row, marked not judged, with the reason. 7. The verdict covers only the product identity and the operating range that were tested. Write untested conditions as outside the approval. 8. Give every approval condition an owner and a due date. A conditional pass becomes final only when all conditions are closed. 9. Attach or cite the laboratory reports; do not restate them.

# Report details

| Report number and revision |  |
| --- | --- |
| Report date (YYYY-MM-DD) |  |
| Customer, site and line |  |
| Trial plan reference and revision |  |
| Prepared by (supplier): name, role |  |
| Approver (customer): name, role |  |
| Laboratory reports cited (numbers) |  |

# 1. Product under evaluation and approval criteria

| Product, grade and lot evaluated |  |
| --- | --- |
| Control or baseline: product and lot, or untreated line |  |

Criteria, copied from the trial plan.

| No. | Criterion | Threshold and direction (unit) | Method | Comparison basis | Fixed on (YYYY-MM-DD) |
| --- | --- | --- | --- | --- | --- |

| Decision rule: met |  |
| --- | --- |
| Decision rule: borderline |  |
| Decision rule: not met |  |
| First result received (YYYY-MM-DD) |  |

[ ]  Every freeze date above is earlier than the first result received.

# 2. Trial conditions and conduct

| Trial period (YYYY-MM-DD to YYYY-MM-DD) |  |
| --- | --- |
| Line and operating state as run |  |
| Samples and controls planned / tested / excluded / lost (count) |  |
| Custody record complete | [ ] yes [ ] no, see deviations |

Operating range actually covered.

| Condition | Unit | Customer operating window (low to high) | Range covered by the run (low to high) |
| --- | --- | --- | --- |

Run validity. Judge each element against the record before reading any result.

| Element | Judgment | Record checked |
| --- | --- | --- |
| Sample identity and custody | [ ] valid [ ] valid with note [ ] not valid |  |
| Method, unit and detection limit | [ ] valid [ ] valid with note [ ] not valid |  |
| Baseline and settling period | [ ] valid [ ] valid with note [ ] not valid |  |
| Control | [ ] valid [ ] valid with note [ ] not valid |  |
| Completeness of results | [ ] valid [ ] valid with note [ ] not valid |  |

Deviations from the trial plan.

| No. | Deviation, as fact | Results affected | Effect on reading |
| --- | --- | --- | --- |

# 3. Results against criteria

| No. | Criterion | Result: mean, SD, n (unit) | Control or baseline (unit) | Margin (%) | Judgment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | [ ] met [ ] borderline [ ] not met [ ] not judged |
|  |  |  |  |  | [ ] met [ ] borderline [ ] not met [ ] not judged |
|  |  |  |  |  | [ ] met [ ] borderline [ ] not met [ ] not judged |
|  |  |  |  |  | [ ] met [ ] borderline [ ] not met [ ] not judged |
|  |  |  |  |  | [ ] met [ ] borderline [ ] not met [ ] not judged |
|  |  |  |  |  | [ ] met [ ] borderline [ ] not met [ ] not judged |

Criteria not judged: criterion, reason, deviation number

Other explanations considered for the results (water, substrate, process, season, measurement change) and the evidence for and against

# 4. Final verdict, qualified scope and approval conditions

| Verdict | [ ] pass [ ] conditional pass [ ] fail |
| --- | --- |
| Criterion the verdict turns on, and its margin (%) |  |
| Consistent with the recommendation the trial was run against | [ ] yes [ ] no, explain: |

Product identity the verdict covers.

| Grade | Specification revision | Manufacturing site | Packaging |
| --- | --- | --- | --- |

Qualified range. Conditions not tested are outside the approval.

| Condition | Unit | Qualified range (low to high) | Outside the approval |
| --- | --- | --- | --- |

| Conditions for regular use: dosing |  |
| --- | --- |
| Conditions for regular use: monitoring |  |
| Conditions for regular use: incoming inspection |  |
| Change notification route |  |

Approval conditions (conditional pass).

| No. | What must still arrive | Owner | Due (YYYY-MM-DD) | Closed (YYYY-MM-DD) |
| --- | --- | --- | --- | --- |

# 5. Other matters (optional)

Only content the approver needs for this decision that fits none of the sections above

# Sign-off

| Role | Name | Signature | Date (YYYY-MM-DD) |
| --- | --- | --- | --- |
| Prepared by (supplier) |  |  |  |
| Reviewed by (supplier) |  |  |  |
| Approved by (customer) |  |  |  |

# Illustrative example (invented values, not a real trial)

This filled excerpt shows how four criteria lead to a conditional pass. The product, thresholds, results and dates are illustrative only; they are not sourced values or recommended limits.

| Product, grade and lot evaluated | Trial coolant, grade X, lot 26-0917 |
| --- | --- |
| Control | Current coolant, lot 26-0802, same line |
| Trial period | 2026-09-22 to 2026-10-20 |
| Decision rule: borderline | Result within the method's repeatability of the threshold, or exactly on it |

| Criterion and threshold | Result and control | Margin (%) | Judgment |
| --- | --- | --- | --- |
| Tool life, at least 110 % of control | 118 % (SD 6 %, n 8); control 100 % | +7.3 | Met |
| Emulsion stability, separation at most 2 mL | 1.2 mL (n 10); control 1.5 mL | +40 | Met |
| Corrosion rating, at most 1 | 1 (n 10); control 0 | 0 | Borderline |
| Bacteria count, at most 10,000 CFU/mL | 6,200 CFU/mL max (9 intervals); control 7,800 CFU/mL | +38 | Met for 9 intervals; 1 not judged (deviation D-04, arrival above the plan temperature range) |

| Verdict | Conditional pass, turns on corrosion (margin 0 %) |
| --- | --- |
| Product identity covered | Grade X, specification revision 4, site A, 1,000 L IBC |
| Qualified range | Concentration 4 to 6 %; above 6 % not tested and outside the approval |
| Approval condition | Repeat corrosion test on the next delivered lot; supplier laboratory; due 2026-11-15 |