# Lubricant failure case record

How to use this template One record covers one failure event on one component. A second failure on the same component gets its own record, linked by case ID. Fill sections 1 to 6 when the failure is reported; fill sections 7 and 8 when the action is taken and again at the review date. Write the cause after the evidence. Write every value with its unit and where it came from. Mark estimates as estimates. Leave a field blank when it applies but was not measured. Write "n/a" when it does not apply. Never fill a blank with a design or nominal value. Keep field names and the failure mode list fixed across sites and years so that cases can be compared. Keep mode, mechanism and cause apart: the mode is what was seen, the mechanism is the physical or chemical process, the cause is the circumstance behind it. Mark the cause certainty: confirmed (other candidates ruled out by a check), inferred (evidence points to it, others not excluded), unconfirmed (hypothesis not tested). Remove customer names and identifying details before sharing the record outside the site. Printing: A4 page with 20 mm side margins, 35 mm bottom margin and the footer 25 mm from the bottom edge, so the footer stays inside a Letter page and clear of most printers' unprintable edge.

# Blank record

## 1. Case identity

| Case ID Unique ID, for example LF-YYYY-NNN |  |
| --- | --- |
| Date detected YYYY-MM-DD |  |
| Site (anonymized) Region and industry, no customer name |  |
| Recorded by Name and role |  |

## 2. Equipment

| Equipment type and tag Equipment class and tag number |  |
| --- | --- |
| Subunit failed Part of the equipment that failed, for example drive train or lubrication system |  |
| Failed component Bearing, gear set, pump, seal or other |  |
| Wetted materials Metals, seals and coatings the fluid touches |  |

## 3. Fluid

| Fluid product Name as on the data sheet |  |
| --- | --- |
| Viscosity grade and base oil type For example ISO VG; mineral, PAO, ester or other |  |
| Known components Only what the TDS, SDS or maker discloses; name the document and revision, or write "not disclosed" |  |
| Hours in service h, since the last oil change |  |
| Date of last oil change YYYY-MM-DD |  |
| Top-ups and mixing Volume (L) and product of each top-up; other products added; or "history unknown" |  |

## 4. Operating conditions

| Oil temperature range °C, and where measured |  |
| --- | --- |
| Load range kW or % of rated, and source |  |
| Speed range rpm, and source |  |
| Operating condition at failure Running, start-up, testing, idle or standby |  |
| Contamination source Water, dust, process fluid, none known |  |
| Source of each value Plant historian, handheld reading on the day, or operator estimate (mark estimates) |  |

## 5. Evidence

| Detection method How the failure was found: oil analysis, vibration, inspection, breakdown or other |  |
| --- | --- |
| Failure mode term ISO 15243 term for rolling bearings; plant or ISO 14224 failure mode list for other equipment |  |
| Viscosity at 40 °C mm2/s, new and used, with test method |  |
| ISO 4406 code Three scale numbers (4/6/14 µm(c)), with test method; "n/a" for grease |  |
| Water content mg/kg, new or baseline and used, with test method |  |
| Acid number mg KOH/g, new and used, with test method |  |
| Other results and limit source Value, unit, method; who set any limit compared against |  |
| Photos and lab reports File names, dates, what each photo shows, report numbers |  |

## 6. Cause and certainty

| Failure mechanism Physical, chemical or other process behind the failure |  |
| --- | --- |
| Failure cause Design, fabrication or installation, operation or maintenance, management, miscellaneous |  |
| Evidence for the cause Which results or checks support it; which candidates were ruled out |  |

Certainty of the cause (tick one):

[ ] Confirmed: a check ruled out the other candidates

[ ] Inferred: evidence points to it, other candidates not excluded

[ ] Unconfirmed: hypothesis not yet tested

Notes:

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

## 7. Action

| Action taken One change per line: what changed |  |
| --- | --- |
| Done by and date Name, YYYY-MM-DD |  |
| Other changes made at the same time List, or "none" |  |
| Review date YYYY-MM-DD, set when the action is recorded |  |

## 8. Outcome

| How long the fix held h or days, to recurrence or to the review date |  |
| --- | --- |
| Recurrence Yes or no |  |
| Linked case IDs Earlier or later records on the same component |  |
| Outcome reviewed by and date Name, YYYY-MM-DD |  |

# Illustrative example (invented values, no real site)

This filled example shows the level of detail expected. It is kept apart from the blank record above; do not copy its values.

## 1. Case identity

| Case ID Unique ID, for example LF-YYYY-NNN | LF-2026-014 |
| --- | --- |
| Date detected YYYY-MM-DD | 2026-09-18 |
| Site (anonymized) Region and industry, no customer name | Site C, paper mill, EU |
| Recorded by Name and role | Technical service engineer |

## 2. Equipment

| Equipment type and tag Equipment class and tag number | Gearbox, GB-12 |
| --- | --- |
| Subunit failed Part of the equipment that failed, for example drive train or lubrication system | Output shaft assembly |
| Failed component Bearing, gear set, pump, seal or other | Output bearing |
| Wetted materials Metals, seals and coatings the fluid touches | Steel, brass cage, NBR seal |

## 3. Fluid

| Fluid product Name as on the data sheet | Gear oil X |
| --- | --- |
| Viscosity grade and base oil type For example ISO VG; mineral, PAO, ester or other | ISO VG 320, mineral |
| Known components Only what the TDS, SDS or maker discloses; name the document and revision, or write "not disclosed" | Sulfur-phosphorus EP (TDS rev. 3) |
| Hours in service h, since the last oil change | 6,200 h |
| Date of last oil change YYYY-MM-DD | 2025-11-04 |
| Top-ups and mixing Volume (L) and product of each top-up; other products added; or "history unknown" | None recorded |

## 4. Operating conditions

| Oil temperature range °C, and where measured | 65 to 78 °C, sump |
| --- | --- |
| Load range kW or % of rated, and source | 180 to 220 kW, historian |
| Speed range rpm, and source | 1,480 rpm, nameplate |
| Operating condition at failure Running, start-up, testing, idle or standby | Running |
| Contamination source Water, dust, process fluid, none known | Washdown water |
| Source of each value Plant historian, handheld reading on the day, or operator estimate (mark estimates) | Temperature and load from historian; speed from nameplate |

## 5. Evidence

| Detection method How the failure was found: oil analysis, vibration, inspection, breakdown or other | Inspection |
| --- | --- |
| Failure mode term ISO 15243 term for rolling bearings; plant or ISO 14224 failure mode list for other equipment | Corrosion, moisture corrosion |
| Viscosity at 40 °C mm2/s, new and used, with test method | 318 new, 296 used (ASTM D445) |
| ISO 4406 code Three scale numbers (4/6/14 µm(c)), with test method; "n/a" for grease | 21/19/15 (ISO 4406) |
| Water content mg/kg, new or baseline and used, with test method | 90 new, 1,800 used (ASTM D6304) |
| Acid number mg KOH/g, new and used, with test method | 0.6 new, 1.1 used (ASTM D664) |
| Other results and limit source Value, unit, method; who set any limit compared against | None |
| Photos and lab reports File names, dates, what each photo shows, report numbers | IMG_0412 (raceway), report 26-3381 |

## 6. Cause and certainty

| Failure mechanism Physical, chemical or other process behind the failure | Corrosion (material failure) |
| --- | --- |
| Failure cause Design, fabrication or installation, operation or maintenance, management, miscellaneous | Operation or maintenance: water ingress through the breather near washdown hoses |
| Evidence for the cause Which results or checks support it; which candidates were ruled out | Water 90 mg/kg new, 1,800 mg/kg used; washdown hoses near breather; breather itself not tested |

[ ] Confirmed

[x] Inferred: the breather itself was not tested

[ ] Unconfirmed

## 7. Action

| Action taken One change per line: what changed | Oil change to the same product; desiccant breather fitted |
| --- | --- |
| Done by and date Name, YYYY-MM-DD | Site maintenance, 2026-09-22 |
| Other changes made at the same time List, or "none" | Two changes at once: outcome cannot be credited to either alone |
| Review date YYYY-MM-DD, set when the action is recorded | 2027-03-02 |

## 8. Outcome

| How long the fix held h or days, to recurrence or to the review date | 2,900 h, no recurrence |
| --- | --- |
| Recurrence Yes or no | No |
| Linked case IDs Earlier or later records on the same component | LF-2025-031 |
| Outcome reviewed by and date Name, YYYY-MM-DD | Technical service engineer, 2027-03-02 |