# Metalworking Fluid Application Survey

Selection checklist for a water-miscible metalworking fluid or a straight oil, filled in before or during a customer visit.

How to fill in this survey 1. Fill in every section before any product name is discussed. Write units as printed in each field. 2. Tag each answer: M = must-have (a fluid that fails it is rejected), P = preference (it only ranks the fluids left). 3. Write who stated each requirement (operations, maintenance, quality, purchasing, EHS, wastewater) and why. 4. Mark a value from memory as "reported"; a measured value gets its method, laboratory and date. 5. Leave nothing blank without a note: write "not asked" or "unknown" and add it to section 9 with who will answer. 6. Attach plant records instead of retyping them: concentration and pH logs, water analysis, permits, current fluid SDS and TDS. 7. Limits for a candidate fluid (water hardness, chloride, concentration range) come from that fluid's technical data sheet; compare them with this survey afterwards. 8. For a straight oil, skip section 4 and the concentration fields in section 5. 9. The example at the end is illustrative and comes from no real plant. Keep it apart from the blank form.

| Customer and site | Prepared by | Visit date | Request |
| --- | --- | --- | --- |
|  |  |  | [ ] First selection [ ] Replacement [ ] Re-check after a change |

# 1. Machines and sump

| Field (unit) | Answer | Tag | Stated by |
| --- | --- | --- | --- |
| Machine type and number of machines |  | [ ] M [ ] P |  |
| Individual sumps or central system |  | [ ] M [ ] P |  |
| Sump or system volume (L) |  | [ ] M [ ] P |  |
| Chip removal and filtration (type, media) |  | [ ] M [ ] P |  |
| Machine age and leak oil (none, some, heavy) |  | [ ] M [ ] P |  |
| Dilution method (premix tank, proportioner, direct to sump) |  | [ ] M [ ] P |  |
| Recycling system in use (yes or no, type) |  | [ ] M [ ] P |  |

# 2. Operations

| Field (unit) | Answer | Tag | Stated by |
| --- | --- | --- | --- |
| Operations, each with alloy (for example drilling, tapping, grinding) |  | [ ] M [ ] P |  |
| Severity per operation (light, moderate, heavy) |  | [ ] M [ ] P |  |
| Tool material and coating |  | [ ] M [ ] P |  |
| Cutting speed or feed where it limits tool life |  | [ ] M [ ] P |  |
| Operator skin contact time (h per shift) |  | [ ] M [ ] P |  |
| Mist control (enclosure, mist collector) |  | [ ] M [ ] P |  |

# 3. Workpiece and machine metals

| Field (unit) | Answer | Tag | Stated by |
| --- | --- | --- | --- |
| Workpiece alloys |  | [ ] M [ ] P |  |
| Machine component metals (aluminum, copper alloys, zinc) |  | [ ] M [ ] P |  |
| Seals, hoses and paint in contact with the fluid |  | [ ] M [ ] P |  |
| Parts storage between operations (days, indoor or outdoor) |  | [ ] M [ ] P |  |

# 4. Mix water (water-miscible fluids)

| Field (unit) | Answer | Tag | Stated by |
| --- | --- | --- | --- |
| Source (mains, well, softened, RO, DI) |  | [ ] M [ ] P |  |
| Hardness (mg/L as CaCO3) |  | [ ] M [ ] P |  |
| Chloride (mg/L) |  | [ ] M [ ] P |  |
| Sulfate (mg/L) |  | [ ] M [ ] P |  |
| pH |  | [ ] M [ ] P |  |
| Conductivity (µS/cm) |  | [ ] M [ ] P |  |
| Laboratory and sample date |  | [ ] M [ ] P |  |
| Can the plant treat the water? (yes or no, how) |  | [ ] M [ ] P |  |

# 5. Current fluid

| Field (unit) | Answer | Tag | Stated by |
| --- | --- | --- | --- |
| Fluid type (straight oil, soluble oil, semisynthetic, synthetic) |  | [ ] M [ ] P |  |
| Target concentration (%) |  | [ ] M [ ] P |  |
| Measured concentration (%), method and refractometer factor |  | [ ] M [ ] P |  |
| Change interval (months) |  | [ ] M [ ] P |  |
| Top-up volume (L per week) |  | [ ] M [ ] P |  |
| Tool life baseline (parts per edge, operation) |  | [ ] M [ ] P |  |
| Problems with dates (odor, foam, residue, rust, skin) |  | [ ] M [ ] P |  |
| Records attached (logs, analyses, SDS, TDS) |  | [ ] M [ ] P |  |

# 6. Restrictions

Ingredient bans, labeling rules and exposure limits. Write the reason and the document behind each one.

| Restriction | Stated by and why | Source document | Tag |
| --- | --- | --- | --- |
|  |  |  | [ ] M [ ] P |
|  |  |  | [ ] M [ ] P |
|  |  |  | [ ] M [ ] P |
|  |  |  | [ ] M [ ] P |
|  |  |  | [ ] M [ ] P |
|  |  |  | [ ] M [ ] P |

# 7. Disposal

Record the route only. Handling steps come from each product's SDS and the rules for the site.

| Field (unit) | Answer | Tag | Stated by |
| --- | --- | --- | --- |
| Waste route (contractor, on-site treatment, discharge) |  | [ ] M [ ] P |  |
| On-site treatment method |  | [ ] M [ ] P |  |
| Discharge permit limits attached (yes or no) |  | [ ] M [ ] P |  |
| Waste treatment tests to run on a candidate fluid |  | [ ] M [ ] P |  |

# 8. Success criteria

A criterion without a threshold or a method is written as "not yet decidable".

| Criterion | Threshold and unit | Method and sampling point | Judged by |
| --- | --- | --- | --- |

# 9. Open items

| Open item | Which candidate judgment it changes | Who answers | By date |
| --- | --- | --- | --- |

# 10. Check before sending

[ ]  Each machine has its type, sump volume in L, chip removal and filtration recorded.

[ ]  The dilution method is named.

[ ]  Every operation has its alloy and a severity.

[ ]  Machine component metals are listed.

[ ]  Operator skin contact time is recorded.

[ ]  Mix water values have units, a laboratory and a sample date.

[ ]  Current fluid concentration shows the target, the measured value and the method.

[ ]  Change interval, weekly top-up and problems with dates are recorded.

[ ]  Each restriction names who stated it and why.

[ ]  The disposal route is named and permit limits are attached.

[ ]  Every answer is tagged M or P.

[ ]  Every success criterion has a threshold, a unit, a method and a judge.

[ ]  Every open item names who will answer it.

# Example (illustrative)

Every machine, value, role and limit below is illustrative and comes from no real plant. Do not copy these values into the blank form.

| Section and field | Answer (illustrative) | Tag | Stated by |
| --- | --- | --- | --- |
| 1. Machines and sump | 6 machining centers, individual sumps, 400 L each, premix tank | P | Maintenance |
| 3. Machine metals | Copper alloy fittings on all six machines | M | Maintenance |
| 4. Mix water | Softened mains, hardness 60 mg/L as CaCO3, chloride 40 mg/L, sulfate 30 mg/L, pH 7.6; plant will not add RO | M | Facilities |
| 5. Current fluid | Semisynthetic, 6% target, 4.5% measured by refractometer, change every 6 months, 120 L top-up per week | n/a | Cell lead |
| 6. Restriction | No boron: wastewater permit excludes it, permit attached | M | Wastewater operator |
| 6. Preference | Lower odor after weekends, no limit given | P | Operators |
| 8. Success criterion | Tool life at least current parts per edge on M8 tapping, counted on 3 taps per machine | Criterion | Cell lead |
| 8. Success criterion | Iron chip corrosion screen per ASTM D4627 at planned concentration | Criterion | Quality |
| 9. Open item | Measured concentration on two machines; permit oil limit | n/a | Cell lead; EHS |