# Plant trial protocol template (industrial chemical products)

Four sections: overview and preparation; conditions, procedure, stop and reversion; measured items and acceptance criteria; owners, schedule, sample custody and reporting.

# Instructions

Use this protocol once the customer has accepted a recommended product and chosen to confirm it by a trial on the line. One protocol per product and line.

Fill the sections in order, starting from the accepted recommendation and its open items. Keep items that documents or lab data already settled out of the trial.

Write every reading, limit and dose with its unit. A value not yet agreed reads "to be confirmed before start"; do not guess it.

Fix the acceptance criteria, stop signs and decision rule before the first trial sample, and keep them fixed after results arrive. A measure added later is reported as an observation, not a criterion.

Take a baseline on the current product on the same line, with the same method and laboratory.

Follow the safety data sheet of each product and the site's change, permit and waste rules in every preparation, changeover, flushing and disposal step.

Treat a run that lost integrity (for example a dosing failure) as a failed run and repeat it rather than interpreting it.

Have production, quality, engineering, safety and the supplier's technical owner sign the protocol before the trial starts.

An illustrative example follows the blank form. It is not a real trial and its values are not recommendations.

# Protocol identification

| Field | Unit or format | Entry |
| --- | --- | --- |
| Protocol number and revision | Text |  |
| Customer site | Text |  |
| Supplier | Text |  |
| Date issued | YYYY-MM-DD |  |

# 1. Trial overview and preparation

| Field | Unit or format | Entry |
| --- | --- | --- |
| Purpose (the one decision the trial informs) | Text |  |
| Accepted recommendation (reference, date) | Text, YYYY-MM-DD |  |
| Product under trial | Name, grade |  |
| Lot under trial | Lot number |  |
| Line or system, and use | Text |  |
| Control or baseline (current product, period) | Text; days |  |
| Samples (trial, control, retained) | Text |  |

Open items from the accepted recommendation (one per line):

| No. | Open item | Measurement that answers it | Result that closes it |
| --- | --- | --- | --- |

Preparation items:

| Preparation item | Owner | Due date | Done |
| --- | --- | --- | --- |
| Safety data sheet of trial product on site |  |  | [ ] |
| Storage and handling as in the safety data sheet |  |  | [ ] |
| Site change approval (management of change) |  |  | [ ] |
| Permits that apply to the trial |  |  | [ ] |
| Dosing equipment checked and calibrated |  |  | [ ] |
| Current product held for reversion |  |  | [ ] |
|  |  |  | [ ] |
|  |  |  | [ ] |

# 2. Trial conditions, procedure, stop and reversion

| Field | Unit or format | Entry |
| --- | --- | --- |
| Dosing conditions | Concentration, dose rate (e.g. mg/L, mL/min, % v/v) |  |
| Trial period | Start and end date; days |  |
| Operating range | Each parameter with unit (e.g. °C, pH, flow) |  |
| Changeover procedure | Steps |  |
| Stabilization window (before first judged sample) | hours or days |  |

Stop signs:

| Stop sign (reading) | Limit or event (with unit) | Who calls the stop (role, name) | Reversion steps |
| --- | --- | --- | --- |

Reversion to the current product (stop, flush or purge, restart, confirm normal range):

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

# 3. Measured items and acceptance criteria

3a. Criterion, threshold and method.

| No. | Criterion | Threshold (with unit) | Test method |
| --- | --- | --- | --- |

3b. Sampling and comparison. Same numbers as 3a.

| No. | Sampling point | Averaging period (days) | Comparator (baseline or control) |
| --- | --- | --- | --- |

Decision rule (pass, fail, extend, and what each requires):

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

# 4. Owners, schedule and result recording

4a. Tasks. The customer's prior approval is the first row.

| What | Who (role, name) | By when (YYYY-MM-DD) | Done |
| --- | --- | --- | --- |
| Customer approves this protocol |  |  | [ ] |

4b. Sample custody. One row per planned sample; confirm each row in the trial log.

| Sample ID | Location | Time (date, hh:mm) | Quantity (mL or g) | Container | Handover (from, to, signature) |
| --- | --- | --- | --- | --- | --- |

4c. Reporting.

| Field | Unit or format | Entry |
| --- | --- | --- |
| Reporting format | Text |  |
| Interim reports (frequency) | Text |  |
| Final report deadline | YYYY-MM-DD |  |
| Report recipients | Names |  |

# Signatures

| Function | Name | Signature | Date |
| --- | --- | --- | --- |
| Production |  |  |  |
| Quality |  |  |  |
| Engineering or maintenance |  |  |  |
| Safety and environment |  |  |  |
| Supplier technical owner |  |  |  |

# Check before signing

[ ]  Every open item has a measurement and a result that closes it.

[ ]  The baseline uses the same line, method and laboratory as the trial.

[ ]  Every stop sign has a limit with unit, a named person and reversion steps.

[ ]  Every criterion has threshold, method, sampling point, averaging period and comparator.

[ ]  The decision rule is written in one sentence for pass, fail and extend.

[ ]  Every planned sample has a custody row, and retained samples are included.

[ ]  Every "to be confirmed before start" item is resolved.

Notes:

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

# Illustrative example (not a real trial)

Every product, value, period, limit and role below is illustrative and comes from no real trial. The values are not recommendations for any system.

## 1. Overview and preparation (illustrative)

| Field | Entry (illustrative) |
| --- | --- |
| Purpose | Decide whether inhibitor X replaces the current product on cooling tower 1 |
| Product and lot | Inhibitor X, lot 2611 |
| Line and use | Open recirculating cooling tower 1, scale and corrosion control |
| Open items | Scale control at the site's makeup water; corrosion on mild steel |
| Baseline | Current product, 14 days before changeover, same sampling and laboratory |
| Preparation | Safety data sheet on site, site change approval, dosing pump calibration, current product tote held |

## 2. Conditions, stop and reversion (illustrative)

| Field | Entry (illustrative) |
| --- | --- |
| Dosing | 15 mg/L as product in makeup water |
| Period | 42 days |
| Operating range | pH 7.8 to 8.6 |
| Stabilization window | First judged sample on day 8 |
| Stop sign 1 | Calcium cycles more than 20 percent below chloride cycles on 2 consecutive days; utilities lead calls it |
| Stop sign 2 | Dosing pump failure over 12 hours; utilities lead calls it |
| Reversion | Stop inhibitor X, restart the current product from the held tote, confirm pH and calcium cycles back in range |

## 3. Acceptance criteria (illustrative)

| Criterion | Threshold | Sampling and averaging | Comparator |
| --- | --- | --- | --- |
| Mild steel coupon corrosion rate | No higher than baseline | Return line coupon rack, coupon exposed 28 days | Baseline |
| Deposit rating on heat exchanger tube | No worse than baseline | Inspection at end of trial | Baseline |

Decision rule (illustrative): pass if both criteria hold; extend 14 days if data are incomplete; otherwise fail.

## 4. Owners, custody and reporting (illustrative)

| Item | Entry (illustrative) |
| --- | --- |
| First row | Customer approves protocol by 2026-11-06 |
| Daily sample | Return line, 08:00, 500 mL, HDPE bottle labelled with date, time and tower; operator to site lab |
| Retained sample | Trial lot 2611, 1 L, kept until the final report is accepted |
| Reporting | Weekly table; final report 7 days after the end of the trial |