# Raw Material Substitution Checklist

For one raw material leaving an existing chemical formulation. One checklist per outgoing material and per formula.

Instructions 1. Fill the sections in order. Each section takes its inputs from the one before it. 2. Record the source of every value (release data, specification, SDS, TDS, CoA). Mark assumptions as assumptions. 3. Tick each [ ] item that applies, and write a reason next to every item marked No. 4. Section 6 (acceptance and stop criteria) is signed before the first bench batch. A criterion changed after results are seen needs a new revision of this checklist. 5. Change one raw material at a time. A change to several materials together is a new design, not a substitution. For a one-time deviation that applies to one work order, complete sections 1 to 4, the release tests in section 5 and their acceptance criteria in section 6; the stability and document items in section 5 can be skipped when they do not apply. 6. This checklist does not replace your change control or management of change procedure, the SDS, or your safety review. Attach it to them. 7. The illustrative example on the last page shows the level of detail. Its values are invented.

| Field | Entry |
| --- | --- |
| Product name and formula ID |  |
| Formula revision |  |
| Checklist revision |  |
| Prepared by |  |
| Date (YYYY-MM-DD) |  |

# 1. Outgoing raw material

| Field | Unit | Entry | Source |
| --- | --- | --- | --- |
| Trade name and grade | text |  |  |
| Supplier | text |  |  |
| CAS number of each constituent (SDS section 3) | CAS number |  |  |
| Amount in formula, as delivered | wt% |  |  |
| Active content of the raw material | % w/w |  |  |
| Amount in formula, as active | wt% |  |  |
| Last date it can be bought | YYYY-MM-DD |  |  |
| Stock on hand | kg |  |  |
| Other formulas that use it | formula IDs |  |  |

Reason it leaves:

[ ]  Discontinuation by the maker

[ ]  Regulation (name the rule and market): ____________________

[ ]  Supply (allocation, lead time, single source)

[ ]  Other: ____________________

# 2. Function of the outgoing material

List every job the material does in the product and in the process. Tie each job to a measured property with its unit and test method.

| Job | Property and test method | Current value and unit | Critical? [ ] Yes [ ] No |
| --- | --- | --- | --- |
| 1. |  |  | [ ] Yes [ ] No |
| 2. |  |  | [ ] Yes [ ] No |
| 3. |  |  | [ ] Yes [ ] No |
| 4. |  |  | [ ] Yes [ ] No |

| Process role | Unit | Entry |
| --- | --- | --- |
| Order of addition (step number) | step |  |
| Batch temperature at addition | °C |  |
| Physical form as delivered (liquid, solid, paste) | text |  |
| Must dissolve before next addition? | [ ] Yes [ ] No |  |
| Components it works with (pairings) | text |  |

# 3. Substitute candidates

Put every candidate on the same rows. Dose on active content: candidate dose (wt%) = outgoing dose (wt%) x outgoing active content (%) / candidate active content (%). Put the difference on water or carrier so the formula sums to 100 wt%.

| Field | Unit | Candidate A | Candidate B | Candidate C |
| --- | --- | --- | --- | --- |
| Trade name, grade, supplier | text |  |  |  |
| CAS number of each constituent (SDS section 3) | CAS number |  |  |  |
| Constituents the outgoing material did not have | text |  |  |  |
| Active content as delivered | % w/w |  |  |  |
| Impurity or byproduct limits in its specification | % or mg/kg |  |  |  |
| Dose matching outgoing active content | wt% |  |  |  |
| TDS received | [ ] Yes [ ] No |  |  |  |
| SDS revision date | YYYY-MM-DD |  |  |  |
| CoAs received (number of lots) | count |  |  |  |
| Regulatory statements per market | market: status |  |  |  |
| Lead time and supply note | weeks; text |  |  |  |

# 4. Compatibility review

Mark each item Pass (P), Caution (C) or Fail (F) per candidate and write the reason.

| Check | A | B | C |
| --- | --- | --- | --- |
| [ ] Each pair with other components (charge, binding, chelation, reaction) |  |  |  |
| [ ] pH window of the product: no hydrolysis or precipitation |  |  |  |
| [ ] Temperature window in process, storage and use |  |  |  |
| [ ] Physical form, dissolution and order of addition |  |  |  |
| [ ] Packaging and materials of construction in contact |  |  |  |
| [ ] Odor, color and appearance |  |  |  |
| [ ] Mixture classification re-checked (EU CLP Article 15 and other markets) |  |  |  |
| [ ] Hazards and handling taken from the candidate's own SDS |  |  |  |

Reasons for any Caution or Fail:

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

# 5. Revalidation scope

Mark Yes or No for each item. Every No needs a reason.

| Item | Yes / No | Conditions or reason |
| --- | --- | --- |
| Release specification test: ____________ | [ ] Y [ ] N |  |
| Release specification test: ____________ | [ ] Y [ ] N |  |
| Release specification test: ____________ | [ ] Y [ ] N |  |
| Shelf-life stability (storage temperature, °C; duration, months) | [ ] Y [ ] N |  |
| Compatibility in use (dilution water, substrates, other products) | [ ] Y [ ] N |  |
| Safety data sheet revision | [ ] Y [ ] N |  |
| Label revision | [ ] Y [ ] N |  |
| Mixture classification evaluation | [ ] Y [ ] N |  |
| Customer notification (per each customer agreement) | [ ] Y [ ] N |  |
| Management of change record (covered process) | [ ] Y [ ] N |  |
| Registrations or notifications per market | [ ] Y [ ] N |  |
| Other: ____________ | [ ] Y [ ] N |  |

# 6. Acceptance criteria, stop criteria and sign-off

One acceptance criterion per reopened test. Fix these before the first bench batch.

| Response | Threshold, unit and direction | Method | Time point |
| --- | --- | --- | --- |

| Stop criterion (observable failure) | Time point | Condition |
| --- | --- | --- |

| Sign-off | Entry |
| --- | --- |
| Criteria approved by (name, role) |  |
| Date approved (YYYY-MM-DD), before batch 1 |  |
| Decision after bench work: [ ] Candidate ___ accepted [ ] Second round [ ] Stop |  |

# Illustrative example (not part of the form)

Every value below is invented to show the level of detail expected. It is not data from a real product, supplier or test.

| Section | Illustrative entry |
| --- | --- |
| 1. Outgoing material | Triethanolamine 99, CAS 102-71-6; 8.0 wt% as delivered, 7.9 wt% as active; reason: supply; last order 2027-03-31 |
| 2. Function | Job 1: pH of 5 % dilution 9.2 (critical). Job 2: neutralizes the fatty acid package into an amine soap that keeps the concentrate clear (critical). Added at step 3, 25 °C. |
| 3. Candidates | A: triethanolamine from a second supplier, CAS 102-71-6, 99.5 % active, dose 7.96 wt%, SDS dated 2026-05-12, 2 CoAs. B: monoisopropanolamine, CAS 78-96-6, dose from titration. |
| 4. Compatibility | A: Pass on all items. B: Caution on soap clarity and odor; own hazard classification, SDS reviewed. |
| 5. Revalidation | Yes: pH, concentrate clarity, emulsion stability, ferrous corrosion test in the specification, 3-month stability at 40 °C, SDS review. No: label for A, reason: same constituent and classification unchanged. |
| 6. Criteria | pH of 5 % dilution 9.0 to 9.5; concentrate clear at 20 °C. Stop: any visible layer at 24 h, 5 °C. Approved by lab lead before batch 1. |