# SDS revision comparison checklist

## Instructions

Use this checklist when a supplier's revised safety data sheet (SDS) arrives for a product already in use. It compares the new version with the version your approval used, section by section, and maps each change to the approvals and documents it reaches.

1. Fill section 1 before reading the new sheet. Confirm that the old version is the one your approval used, not simply the latest file in the folder.

2. In section 2, compare the header and all 16 sections. Enter old and new values, tick one status and say whether the change reaches an approval. An empty or unclear section is Unknown.

3. In section 3, list each approval or document that relied on the old sheet, write the basis it used, and tick one verdict.

4. In section 4, send the supplier questions in writing, assign each action an owner and a date, and tick the distribution items.

5. Sign off when every Unknown is closed or carried as an action with an owner.

This checklist compares two documents. It does not show components a sheet does not list, and it does not replace a requalification trial when a change reaches a performance result.

The blank form comes first. The illustrative example after it uses invented values to show a filled checklist; it is not data about any real product, supplier or customer.

# 1. Document identity

| Product name and code |  |
| --- | --- |
| Supplier |  |
| Format of the sheet (jurisdiction) | [ ] US OSHA [ ] EU REACH Annex II [ ] Canada WHMIS [ ] Other: ____ |
| Language |  |
| Old version: number, revision date, file name |  |
| Is the old version the one your approval used? | [ ] Yes [ ] No |
| New version: number, revision date, supersedes |  |
| Received: date and how (shipment, email, portal) |  |
| Where the revision note sits | [ ] Section 16 [ ] Cover letter [ ] Elsewhere [ ] None |

# 2. Section-by-section comparison

Status: Same = both versions give the same value. Changed = the value differs. Unknown = the section is missing, unclear or not comparable. Reaches approval: does the change touch a finding an existing approval or workplace document relied on? Tick both boxes for every row, including rows with no change. Match components by CAS number and concentration, not by name.

| SDS section (what to compare, unit) | Old version | New version | Status | Reaches approval |
| --- | --- | --- | --- | --- |
| Header (Version number; revision date (YYYY-MM-DD); version superseded) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 1. Identification (Product identifier, other identifiers, recommended use and restrictions, supplier, emergency number, UFI (EU)) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 2. Hazard identification (Hazard classes and categories, signal word, pictograms, H and P statements) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 3. Composition (Each component: name, CAS number, concentration (% by weight) or range; EU: specific concentration limit, M-factor, acute toxicity estimate) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 4. First aid measures (Exposure routes, symptoms, medical attention) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 5. Firefighting measures (Extinguishing media, combustion hazards) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 6. Accidental release measures (Spill precautions, containment, cleanup) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 7. Handling and storage (Storage conditions (°C), incompatible materials) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 8. Exposure controls and personal protection (Exposure limits per component (mg/m3 or ppm), controls, protective equipment) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 9. Physical and chemical properties (Flash point (°C), pH, density (g/cm3), viscosity (mm2/s), with test methods) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 10. Stability and reactivity (Incompatible materials, conditions to avoid, decomposition products) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 11. Toxicological information (Acute toxicity values (mg/kg), routes, health effects) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 12. Ecological information (Ecotoxicity (mg/L), persistence, mobility) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 13. Disposal considerations (Waste treatment methods) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 14. Transport information (UN number, proper shipping name, transport class, packing group) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 15. Regulatory information (Rules and listings that apply to the product) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |
| 16. Other information (Revision date, indication of where changes were made, abbreviations) |  |  | [ ] Same [ ] Changed [ ] Unknown | [ ] Yes [ ] No |

# 3. Effect on existing approvals

List every approval or document that relied on the old sheet. Verdict: Holds = the change touches nothing in its basis. Re-check = the change touches a use condition (storage, protective equipment, transport). Re-evaluate = the change touches the basis itself (composition, a property outside the approval limit, a hazard class the approval excluded).

| Approval or document | Basis it used (section, value) | Changed section that reaches it | Verdict |
| --- | --- | --- | --- |
|  |  |  | [ ] Holds [ ] Re-check [ ] Re-evaluate |
|  |  |  | [ ] Holds [ ] Re-check [ ] Re-evaluate |
|  |  |  | [ ] Holds [ ] Re-check [ ] Re-evaluate |
|  |  |  | [ ] Holds [ ] Re-check [ ] Re-evaluate |
|  |  |  | [ ] Holds [ ] Re-check [ ] Re-evaluate |
|  |  |  | [ ] Holds [ ] Re-check [ ] Re-evaluate |

# 4. Follow-up actions

Questions to the supplier (send in writing, record the answer and its date):

[ ] Which sections changed, and why: new hazard data, classification rule change, reformulation, raw material or manufacturing site change, or format only?

Answer and date: ____________________________________________________________

[ ] Was the product reformulated, including components not listed in section 3?

Answer and date: ____________________________________________________________

[ ] From which lot or production date does the new version apply?

Answer and date: ____________________________________________________________

[ ] Did the technical data sheet, specification or certificate of analysis limits change at the same time?

Answer and date: ____________________________________________________________

[ ] Does the revision apply in every market, or only to the jurisdiction this sheet is written for?

Answer and date: ____________________________________________________________

Actions:

| What | Who | By when (YYYY-MM-DD) | Done (date) |
| --- | --- | --- | --- |

[ ] Workplace copy replaced with the new version

[ ] Old version archived with its dates, not deleted

[ ] New version sent to your own customers where you supply the product (for example under REACH Article 31(9) or OSHA 29 CFR 1910.1200(g)(6))

# Sign-off

| Role | Name and function | Date (YYYY-MM-DD) | Signature |
| --- | --- | --- | --- |
| Compared by |  |  |  |
| Approved by |  |  |  |

# Illustrative example (invented values, not real data)

A water-miscible cutting fluid moves from SDS version 3.0 to version 4.1. Every value below is illustrative.

# 1. Document identity

| Product name and code | Cutting fluid X, CF-200 |
| --- | --- |
| Supplier | Supplier Y |
| Format of the sheet (jurisdiction) | [x] US OSHA [ ] EU REACH Annex II [ ] Canada WHMIS [ ] Other: ____ |
| Language | English |
| Old version: number, revision date, file name | 3.0, 2025-11-02, CF-200_SDS_v3.0.pdf |
| Is the old version the one your approval used? | [x] Yes [ ] No |
| New version: number, revision date, supersedes | 4.1, 2026-03-20, supersedes 3.0 |
| Received: date and how (shipment, email, portal) | 2026-04-02, with shipment |
| Where the revision note sits | [x] Section 16 [ ] Cover letter [ ] Elsewhere [ ] None |

# 2. Section-by-section comparison

Status: Same = both versions give the same value. Changed = the value differs. Unknown = the section is missing, unclear or not comparable. Reaches approval: does the change touch a finding an existing approval or workplace document relied on? Tick both boxes for every row, including rows with no change. Match components by CAS number and concentration, not by name.

| SDS section (what to compare, unit) | Old version | New version | Status | Reaches approval |
| --- | --- | --- | --- | --- |
| Header | 3.0, 2025-11-02 | 4.1, 2026-03-20, supersedes 3.0 | Changed | No |
| 1. Identification | Cutting fluid X, CF-200; supplier Y | Same | Same | No |
| 2. Hazard identification | Skin irritation category 2 | Skin irritation 2; eye irritation 2 | Changed | Yes |
| 3. Composition | Three listed components; third 1 to 5 % | Same CAS numbers; third 1 to 3 % | Changed | Yes |
| 4. First aid measures | Skin and eye rinsing text | Same text | Same | No |
| 5. Firefighting measures | Water spray, foam, dry powder | Same | Same | No |
| 6. Accidental release measures | Absorb with inert material | Same | Same | No |
| 7. Handling and storage | Store 5 to 40 °C | Store 5 to 40 °C | Same | No |
| 8. Exposure controls and personal protection | Nitrile gloves | Nitrile gloves; safety goggles | Changed | Yes |
| 9. Physical and chemical properties | pH 9.0 to 9.6 at 5 % dilution | pH 9.0 to 9.6 at 5 % dilution | Same | No |
| 10. Stability and reactivity | Avoid strong acids | Same | Same | No |
| 11. Toxicological information | Skin irritation data | Same | Same | No |
| 12. Ecological information | No data available | No data available | Same | No |
| 13. Disposal considerations | Dispose under local rules | Same | Same | No |
| 14. Transport information | Not regulated for transport | Not regulated for transport | Same | No |
| 15. Regulatory information | Regulatory listings named | Same listings | Same | No |
| 16. Other information | First issue note | Sections 2, 3 and 8 revised | Changed | No |

# 3. Effect on existing approvals

List every approval or document that relied on the old sheet. Verdict: Holds = the change touches nothing in its basis. Re-check = the change touches a use condition (storage, protective equipment, transport). Re-evaluate = the change touches the basis itself (composition, a property outside the approval limit, a hazard class the approval excluded).

| Approval or document | Basis it used (section, value) | Changed section that reaches it | Verdict |
| --- | --- | --- | --- |
| Customer trial approval, aluminium machining, 2026-03-30 | Composition of version 3.0; pH | Section 3 | [ ] Holds [ ] Re-check [x] Re-evaluate |
| Secondary container labels | Section 2 hazard statements | Section 2 | [ ] Holds [x] Re-check [ ] Re-evaluate |
| Workplace exposure assessment and PPE rule | Section 8 protective equipment | Section 8 | [ ] Holds [x] Re-check [ ] Re-evaluate |
| Storage area setup | Section 7 storage temperature | None | [x] Holds [ ] Re-check [ ] Re-evaluate |

# 4. Follow-up actions

Questions to the supplier (send in writing, record the answer and its date):

[x] Which sections changed, and why: new hazard data, classification rule change, reformulation, raw material or manufacturing site change, or format only?

Asked 2026-04-03; answer: new eye irritation data and narrowed range, no new component

[ ] Was the product reformulated, including components not listed in section 3?

Asked 2026-04-03; answer pending

[x] From which lot or production date does the new version apply?

Answer: from lot 2603-07

[x] Did the technical data sheet, specification or certificate of analysis limits change at the same time?

Answer: no change

[x] Does the revision apply in every market, or only to the jurisdiction this sheet is written for?

Answer: EU and US sheets both revised

Actions:

| What | Who | By when (YYYY-MM-DD) | Done (date) |
| --- | --- | --- | --- |
| Re-evaluate the trial approval for the narrowed range | Technical service | 2026-04-17 |  |
| Reprint secondary labels with eye irritation statement | EHS officer | 2026-04-10 | 2026-04-09 |
| Add safety goggles to the workplace PPE rule | EHS officer | 2026-04-10 | 2026-04-09 |

[x] Workplace copy replaced with the new version

[x] Old version archived with its dates, not deleted

[x] New version sent to your own customers where you supply the product (for example under REACH Article 31(9) or OSHA 29 CFR 1910.1200(g)(6))

# Sign-off

| Role | Name and function | Date (YYYY-MM-DD) | Signature |
| --- | --- | --- | --- |
| Compared by | Product steward | 2026-04-03 |  |
| Approved by | Quality manager | 2026-04-17 |  |