# Shelf life retest checklist for industrial chemicals

One form per lot. Product: ____________________   Lot: ____________   Form date (YYYY-MM-DD): ____________

## Instructions

Use this checklist when a stored lot of an industrial chemical has passed, or is about to pass, its shelf life or retest date.

Fill the sections in order, 1 to 5. Fix the retest items and their acceptance limits in section 4 before the sample reaches the laboratory.

Copy product, lot and dates from the label and the Certificate of Analysis (CoA) of this lot, not from the general product data sheet.

Write "Not known" where no record exists. Do not leave a field blank.

Take acceptance limits from the original product specification. If only the CoA is held, enter its values as reference and mark the limit "Unconfirmed" until the manufacturer states it.

Read the safety data sheet (SDS) sections 7, 8, 10 and 13 before sampling, handling or disposal, and follow the site PPE rule. Reactive, peroxide forming, self-reactive or oxidizing products are outside this checklist: follow the SDS and the manufacturer instead.

Only the manufacturer can extend the shelf life or recertify the lot. The user decides on use, based on the retest result. Record both decisions in section 5.

The filled example on the last page is illustrative. Its values describe no real product, supplier or laboratory.

## 1. Product and lot

| Field | Unit or format | Entry |
| --- | --- | --- |
| Product name | Text |  |
| Grade | Text |  |
| Manufacturer | Text |  |
| Lot number | Text |  |
| Container type and count | Text; number |  |
| Quantity held | kg or L |  |
| Manufacture date | YYYY-MM-DD |  |
| Stated shelf life | months |  |
| Expiry or retest date on label or CoA | YYYY-MM-DD |  |
| CoA of this lot held | Yes / No |  |
| Original specification held (document, revision) | Text |  |

## 2. Storage history

| Field | Unit or format | Entry |
| --- | --- | --- |
| Storage location | Text |  |
| Minimum temperature seen | °C |  |
| Maximum temperature seen | °C |  |
| Source of temperature data | Logger / building records / estimate |  |
| Storage conditions stated in SDS section 7 | Text |  |

[ ]  Containers stayed sealed (if opened: dates and number of openings in notes)

[ ]  Product stayed in the original manufacturer-approved packaging

[ ]  Kept out of direct sunlight

[ ]  Kept in a climate controlled area, free of extreme temperature and humidity

[ ]  No freezing or freeze and thaw cycles (if not known, tick Not known below)

[ ]  Storage within the SDS section 7 conditions

[ ]  Containers inspected: no damage, leaks, bulging or label loss

[ ]  Not known: one or more items above have no record

Notes:

______________________________________________________________________________________

______________________________________________________________________________________

## 3. Degradation mechanism

Tick each mechanism that applies to this product class. Use SDS section 10 (stability and reactivity) and the manufacturer's information.

[ ]  Oxidation (oils, opened containers)

[ ]  Water uptake (hygroscopic products, opened or unsealed containers)

[ ]  Hydrolysis (ester based fluids, products stored with water present)

[ ]  Oil or phase separation (greases, emulsions, dispersions)

[ ]  Freeze and thaw damage (water-borne products, emulsions)

[ ]  Microbial growth (water-miscible fluids)

[ ]  Other (describe in notes)

[ ]  Reactive, peroxide forming, self-reactive or oxidizing product: stop and refer to the SDS and the manufacturer

SDS used (revision date): ______________________________

Notes:

______________________________________________________________________________________

______________________________________________________________________________________

## 4. Retest items and acceptance limits

Fill columns 1 to 4 before testing. Enter columns 5 and 6 from the laboratory report.

| Property | Method and edition | Acceptance limit and unit (source, revision) | Original CoA value | Retest result | Within limit? |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | [ ] Yes [ ] No |
|  |  |  |  |  | [ ] Yes [ ] No |
|  |  |  |  |  | [ ] Yes [ ] No |
|  |  |  |  |  | [ ] Yes [ ] No |
|  |  |  |  |  | [ ] Yes [ ] No |
|  |  |  |  |  | [ ] Yes [ ] No |
|  |  |  |  |  | [ ] Yes [ ] No |

| Field | Unit or format | Entry |
| --- | --- | --- |
| Sampling: containers sampled out of total | number / number |  |
| Sample date | YYYY-MM-DD |  |
| Laboratory | Text |  |
| Laboratory report reference and date | Text; YYYY-MM-DD |  |

[ ]  Limits taken from the original specification before testing

[ ]  Limits marked Unconfirmed where only CoA values were available

[ ]  Same method and edition as the original specification, or difference noted

[ ]  Each container sampled, or sampling plan noted

[ ]  Protection worn as listed in SDS section 8 and required by the site PPE rule

## 5. Decision

Manufacturer statement

[ ]  Shelf life extended

[ ]  Lot recertified

[ ]  Extension refused

[ ]  No answer received

| Field | Unit or format | Entry |
| --- | --- | --- |
| New retest or expiry date | YYYY-MM-DD |  |
| Manufacturer contact and statement reference | Text |  |
| Statement date | YYYY-MM-DD |  |

User decision

[ ]  Use

[ ]  Restricted use (describe)

[ ]  Return to supplier

[ ]  Disposal under SDS section 13 and local waste rules

| Field | Unit or format | Entry |
| --- | --- | --- |
| Decided by (name, role) | Text |  |
| Decision date | YYYY-MM-DD |  |
| Attached: laboratory report, storage records, manufacturer statement | Yes / No |  |

Notes:

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

## Example (illustrative): filled checklist

All values on this page are illustrative. They describe no real product, supplier or laboratory. Do not use them as limits.

| Section | Entry (illustrative) | Result or reading (illustrative) |
| --- | --- | --- |
| 1. Product and lot | Hydraulic oil ISO VG 46, lot 24-0815, 4 drums of 200 L, 800 L held, made 2024-08-15, shelf life 24 months, expired 2026-08-15 | CoA of this lot held; specification revision 3 held |
| 2. Storage history | Indoor warehouse, 5 °C to 32 °C from a logger, drums sealed, original packaging, no sunlight | Within SDS section 7 conditions |
| 3. Mechanisms | Oxidation, water uptake | No ester content, so hydrolysis not ticked |
| 4. Retest item | Kinematic viscosity at 40 °C, limit 41.4 to 50.6 mm2/s from the specification | 46.1 mm2/s, within limit |
| 4. Retest item | Acid number, limit from the specification | 0.4 mg KOH/g, within limit |
| 4. Retest item | Water content, limit from the specification | 120 mg/kg, within limit |
| 5. Decision | Manufacturer issued a new retest date of 2027-08-15 | User approved for use; both decisions named and dated |