# Trial verdict decision chart

## Instructions

Use this chart and form to judge an industrial product trial as pass, conditional pass, fail, not judged or invalid run, and to state the scope the verdict covers.

Fill section 1 of the form before the trial starts and have both parties sign it, so the date the criteria were fixed is on record.

Fill section 2 while the run is in progress, from the sample log, laboratory reports and line records, not from memory at the end.

Work the 11 steps below in order once sections 1 and 2 and the results in section 3 are complete. Steps 1 to 6 judge the run, steps 7 to 9 judge each criterion, steps 10 and 11 set the verdict and its scope. Tick yes or no in the Answer column.

Write unconfirmed in a cell when a record is missing; do not leave it blank. Do not change a criterion after a result is seen: that is a new run.

The example at the end is illustrative only and is kept apart from the blank form.

## Decision steps

| Step | Question | If yes | If no | Answer |
| --- | --- | --- | --- | --- |
| 1. Criteria fixed | Was every criterion, its threshold with unit, its test method and the decision rule written and dated before the first result? | Go to step 2 | Not judged: record why, freeze criteria, plan a new run | [ ] yes [ ] no |
| 2. Run identity | Are product, lot, line or unit, sample points and trial period recorded? | Go to step 3 | Not judged until identity is confirmed | [ ] yes [ ] no |
| 3. Sample custody | Is each sample traceable from collection to test with no gap or substitution? | Go to step 4 | Invalid run if a decisive criterion depends on the affected samples; otherwise exclude those results and judge their criteria unassessed at step 6 | [ ] yes [ ] no |
| 4. Method as planned | Was each result measured by the planned method, unit and detection limit? | Go to step 5 | Invalid result; record the deviation and its effect | [ ] yes [ ] no |
| 5. Baseline and control | Is there a baseline period or a control run under the same conditions? | Go to step 6 | Not judged for criteria that compare against baseline or control | [ ] yes [ ] no |
| 6. Completeness | Does every criterion have its planned number of independent results? | Go to step 7 | Unassessed for that criterion | [ ] yes [ ] no |
| 7. Clear margin | Does the result clear its threshold by more than the method precision or agreed guard band? | Met | Go to step 8 | [ ] yes [ ] no |
| 8. Inside the band | Is the result within the method precision or guard band of the threshold? | Borderline | Not met | [ ] yes [ ] no |
| 9. Other causes | Are causes outside the product (water, substrate, process, season, measurement change) ruled out? | Keep the judgment | Lower the confidence; record the open cause | [ ] yes [ ] no |
| 10. Verdict | Are all criteria met? | Pass, within the tested scope | Fail if a decisive criterion is not met; otherwise not judged if a decisive criterion is unassessed; otherwise conditional pass, with each borderline or unassessed criterion and each non-decisive criterion not met named as an approval condition | [ ] yes [ ] no |
| 11. Scope | Did the run cover every condition of regular use? | Qualified for that window | Qualified for the tested range only | [ ] yes [ ] no |

# Trial verdict form

## 1. Product under evaluation and approval criteria

Trial title: ____________________________________________________________

Product name, grade and supplier: ____________________________________________________________

Lot number(s) evaluated: ____________________________________________________________

Prepared by (name, organization): ____________________________________________________________

One row per criterion. Write the threshold with its unit and direction (at least, at most, range). Comparison basis: absolute value, change against baseline, or difference from control.

| Criterion | Threshold and unit | Test method | Quantitation limit and unit | Comparison basis | Decisive? (yes/no) |
| --- | --- | --- | --- | --- | --- |

Decision rule for results near a threshold (method precision or guard band, with unit): ____________________________________________________________

_____________________________________________________________________________________

Date criteria were fixed (YYYY-MM-DD): ____________________________________________________________

Signed by (both parties, name and date): ____________________________________________________________

## 2. Trial conditions and conduct

Trial period as run, start and end (YYYY-MM-DD): ____________________________________________________________

Line, unit or machine: ____________________________________________________________

Operating state as run (load, speed, temperature, concentration, with units): ____________________________________________________________

Sampling as run (sample points, frequency, number of samples): ____________________________________________________________

Baseline period or control run (what, when): ____________________________________________________________

Mark each element valid, deviated or unconfirmed, and name the record it was checked against. Write a deviation as the fact, the results it affects and its effect on the reading.

| Element | Valid / deviated / unconfirmed | Record checked | Deviation and results affected |
| --- | --- | --- | --- |
| Sample identity and custody |  |  |  |
| Method, unit and detection limit |  |  |  |
| Baseline and transition period |  |  |  |
| Control |  |  |  |
| Result completeness |  |  |  |

Results excluded from the judgment, and why: ____________________________________________________________

_____________________________________________________________________________________

## 3. Results against criteria

Use values from independent samples, not analytical replicates of one sample. Margin = result minus threshold where a higher result passes, threshold minus result where a lower result passes, in the criterion's unit, so a positive margin reads as passing. A criterion without a valid result keeps its row, with no margin, judged unassessed.

| Criterion | Mean and unit | Standard deviation | n (independent) | Control or baseline value | Threshold | Margin | Judgment (met / borderline / not met / unassessed) |
| --- | --- | --- | --- | --- | --- | --- | --- |

Other causes outside the product. Note the evidence for and against an effect on the results.

| Possible cause | Evidence for | Evidence against | Ruled out? (yes/no) |
| --- | --- | --- | --- |
| Water or make-up quality |  |  |  |
| Substrate or material change |  |  |  |
| Process or equipment change |  |  |  |
| Season or ambient conditions |  |  |  |
| Measurement or laboratory change |  |  |  |

## 4. Final verdict, qualified scope, and approval conditions

Verdict: [ ] pass  [ ] conditional pass  [ ] fail  [ ] not judged  [ ] invalid run

Criterion the verdict turns on, with its margin and unit: ____________________________________________________________

Qualified product identity, so that a later delivery can be matched to the trial lot.

Grade and specification revision: ____________________________________________________________

SDS revision: ____________________________________________________________

Manufacturing site: ____________________________________________________________

Packaging: ____________________________________________________________

Certificate of Analysis of the trial lot (reference): ____________________________________________________________

Qualified scope. One row per operating condition, in its own unit. An untested condition stays outside the qualified scope; an extrapolated range is not qualified.

| Condition and unit | Tested low | Tested high | Customer window low | Customer window high | Extrapolated range (not qualified) |
| --- | --- | --- | --- | --- | --- |

Conditions for regular use.

Dosing or concentration in regular use (with unit): ____________________________________________________________

Monitoring items and frequency: ____________________________________________________________

Incoming inspection items: ____________________________________________________________

Change notification route: ____________________________________________________________

Approval conditions. For a conditional verdict, name each gap, its effect and the measurement that must still arrive before the verdict is final. A condition not yet agreed keeps its row, marked to be agreed.

| Gap or condition | Effect on verdict, confidence or scope | Measurement that would make the verdict final | Owner | Due date |
| --- | --- | --- | --- | --- |

Verdict signed by (name, role, date): ____________________________________________________________

Verdict accepted by (name, role, date): ____________________________________________________________

# Example (illustrative only)

Every product, value, threshold and date below is invented to show how the form reads. None comes from a real trial.

| Section | Illustrative entry |
| --- | --- |
| 1. Product and criteria | Metalworking coolant X, grade A, lot 2410-07. Tool life change against control at least 10 % (decisive), agreed guard band 2 percentage points; foam height after standing at most 50 mL, method precision 10 mL; no staining on cast iron chips. Decision rule: a result within the guard band or method precision of its threshold is borderline. Fixed 2026-02-20, signed by both parties. |
| 2. Conditions and conduct | Machining line 3, 2026-03-02 to 2026-04-10, concentration 5 to 7 %, steel grades only. Control: unchanged line on the old coolant throughout. Custody valid (sample log); method valid; control valid; completeness deviated: cast iron not machined, so no corrosion result. |
| 3. Results against criteria | Tool life 14 %, SD 3 %, n 6 machines, margin 4 percentage points, above the 2 point guard band: met. Foam 48 mL, margin 2 mL, within the 10 mL precision: borderline. Cast iron staining: no result, unassessed. Tool supplier and grades unchanged: ruled out. |
| 4. Verdict, scope and conditions | Conditional pass, turning on foam (margin 2 mL). Qualified for steel grades and 5 to 7 % concentration; customer window 4 to 8 %; cast iron outside the scope. SDS revision 4, specification revision C. Approval conditions: repeat foam test in winter make-up water (owner QA); cast iron corrosion test (owner process engineering). |